Georgia Transportation Institute

Luncheon Seminar Series

Advances in Discrete Choice Modeling

by Prof. Frank S. Koppelman

Northwestern University

Date:                October 3, 2007 from 11 to 12 AM

Location:          Mason 311 (Civil Engineering Building, Map it!)

We hope to see you in this second talk of the series! Pizza and soft drinks will be provided.

Abstract

Random utility maximization discrete choice models are widely used in transportation and other fields to represent the choice among a set of alternatives. The decision maker is assumed to choose the alternative with the highest utility.  The utility of an alternative to the decision maker is represented by a deterministic portion which is a function of the attributes of the alternative and the characteristics of the decision-maker and an additive random component which represents unknown and/or unobservable components of the decision maker’s utility function.

Early development of choice models was based on the assumption that the error terms were either multivariate normal or independently and identically Type I extreme value (gumbel) distributed.  The multivariate normal assumption leads to the multinomial probit (MNP) model; the independent and identical gumbel assumption leads to the multinomial logit (MNL) model.  The probit model allows great flexibility in the structure of the error terms and the distribution of model parameters but estimation and application requires numerical integration of a multi-dimensional normal distribution.  The multinomial logit probabilities have a closed form but the assumption that the error terms are independently and identically distributed across alternatives and cases (individuals, households or choice repetitions) and the use of a fixed set of parameters for the entire population places important limitations on the model.  Developments in the structure of discrete choice models have been directed at reducing the computational burden of the multinomial probit model or increasing the flexibility of logit type models.  This seminar describes the increasing flexibility of logit models. This culminates with the Formulation of the Generalized Extreme Value (GEV), Mixed Logit (MXL) and Mixed GEV (MGEV) Models which combine the advantages of logit and probit models while limiting the degree of computational complexity associated with the probit model.

Bio

FRANK S. KOPPELMAN is Professor of Civil Engineering and Transportation at Northwestern University where he has taught since 1975.  His research addresses the development and application of advanced and activity based models of travel demand integrating econometric and market research methods to enhance understanding of travel behavior for both urban and intercity travel.  Koppelman has led numerous research studies into these and other aspects of travel behavior analysis and travel model development.  He has developed advanced logit models that allow highly flexible substitution relationships between pairs of alternatives. He has also worked with consultant teams and peer review panels on a variety of advanced urban and intercity travel demand modeling studies including Activity Based Model Development in the Houston-Galveston and San Francisco Bay Areas.

Koppelman is lead author of the recently released A Self Instructing Course in Mode Choice Modeling: Multinomial and Nested Logit Models; go to

http://www.civil.northwestern.edu/people/koppelman/PDFs/LM_Draft_060131Final-60630.pdf

He is the first recipient of the Lifetime Achievement Award of the International Association for Travel Behavior Research.  He holds a PhD and BS in Civil Engineering from the Massachusetts of Technology (MIT) and an MBA from the Harvard University Graduate School of Business Administration (HBS).  He is active in the Transportation Research Board where he is past-Chairman and Member Emeritus of the Committee on Travel Demand Analysis and Forecasting and has served on a number of standing and special committees.  He is past Associate Editor of Transportation Research-B.

-----

For questions, comments or suggestions please contact Jorge.Laval@ce.gatech.edu.